# Master thesis projects for Professor Sigurd Skogestad. 2009

1. Stabilization of two-phase flow in risers from reservoirs (anti-slug control)

(co-supervisor: from StatoilHydro)

This project is motivated probelsm with riser slugs in offshore fields in the North Sea. Based on a simplified model of the process the objectives are the following

1. Tune the simple model so that it represents the actual behavior

2. Analyze the simple model, for example, finding the possible steady-states and by linearization and controllability analysis of the desired flow regime

3. Discuss the possibility for avoiding the slug flow, for example, by use of active control.

2.     Simulering, optimal drift og selvoptimaliserende regulering av LNG-anlegg

Produksjonen av LNG skjer ved at naturgass kjøles i et nettverk av kjølesykluser før den blir lagret flytende ved atmosfærisk trykk (volumet er da redusert mer enn 600 ganger). Dette krever at naturgassen kjøles ned til omlag -162 C, noe som er en svært energikrevende prosess. For å redusere energibehovet har det blitt utviklet komplekse kjølenettverk som reduserer de termodynamiske tapene i varmevekslerne. 
Disse avanserte designene gjør at prosessen får flere manipulerte pådrag og større interaksjoner, noe som fører til reguleringsproblemer.

Prosessen Mixed Fluid Cascade (MFC) er utviklet av et Statoil/Linde-samarbeid.

For en slik komplisert prosess er det viktig å ha en systematisk fremgangsmåte for å bestemme hva som skal reguleres.

Begrepet selvoptimaliserende regulering betyr at man ved å holde et sett 
av regulerte variabler konstante oppnår nær-optimal drift for alle 
forventede forstyrrelser.

Prosjektoppgaven vil for eksempel kunne gå ut på å lage en modell av en enkel LNG-prosess i MATLAB eller gPROMS, for så å simulere prosessen dynamisk og studere hvordan ulike valg av regulering fungerer for denne prosessen. Det vil også være gode muligheter for å bygge videre på oppgaven i en diplomoppgave til våren.

Dette er en meget aktuell oppgave siden 
Snøhvit-feltet er i oppstartfasen. Vi har et meget godt samarbeid med 
StatoilHydro og med ABB.

Medveileder: Magnus G. Jacobsen

4. Regulering av Kaibel destillasjonskolonne.

Petlyuk-kolonner tas i stigende bruk i verden pga. mulighetene for reduksjoner i energi og investering. Vi har en flott eksperimentell rigg i hall C. Foreløpig kjører vi alkoholer, men vi planlegger et samarbeid med Statoil på kjøring av C5-C6 blandinger. Det arbeides også med å legge inn automatisk regulering av dampsplitten som vil være helt nytt når det gjelder denne type . Medveileder: Ibar J. Halvorsen, SINTEF

5. Stabilization of two-phase flow with parallell branches

(medveileder: Magnus Glosli Jacobsen)

This project is motivated by serious operational problems encountered at StatoilHydro’s new LNG facility in Hammerfest. Based on a simplified model of the process the objectives are the following

1. Tune the simple model so that it represents the actual behavior

2. Analyze the simple model, for example, finding the possible steady-states and by linearization and controllability analysis of the desired flow regime

3. Discuss the possibility for avoiding the operational problems, for example, by use of active control.

6. Explicit MPC

(medveileder: Henrik Manum)

For systems with a small number of states the model predictive control (MPC)

problem may be solved off-line giving an optimal control law as a function of

the present state. The solution is such that the state space will be divided

into regions with different optimal state feedback laws. A key issue in

implementation of an explicit MPC controller is to identify the region that

contains the current state. Previous work on this topic includes utilization

of a binary search three throughout the regions. If not all states are

measured a state observer needs to be included in the loop.

Recent work in the control group has showed that in can be possible to avoid

both the binary search three and the state observer by finding invariant

combinations of measurements in each region that are such that when controlled

at constant setpoint yields zero loss from optimality. These variable

combinations serves both f2or optimal control and identification of the current

region.

A project in this area will consist of at least these two topics:

1.) Compare measurement feedback with state estimation from a robust

control point-of-view. This means finding calculation of performance

and robustness measures for both techniques.

2.) Compare the computational load of using a binary search tree to

the present method which will be described in more detail to

prospective students.

7. Dynamisk simulering av prosess med alternative reguleringsstrukturer

Medveiledere: Mehdi Panahi og Ramprasad Yelshuru

Medveileder Honeywell: Bjørn Einar Bjartnes

Oppgaven tar utgangspunkt i bruk av en dynamisk prosess-simulator (Hysys) og Honeywells MPC software (Profit Controller) som er integrert i UniSims Profit Controller. Målet for oppgaven vil være å lage et undervisningsopplegg for industrielle anvendelser av MPC i en dynamisk prosessimulator. Arbeidet vil bestå av dynamisk prosessmodellering, modellidentifisering og kontrollerdesign, samt å lage et øvingsopplegg med ferdige UniSim modeller og kontrollstrukturer som kan benyttes videre i undervisning.

Kursmateriale for egenopplæring i UniSim er tilgjengelig fra Honeywell.

Linker til beskrivelse av software:

http://hpsweb.honeywell.com/Cultures/en-US/Products/ControlApplications/AdvancedControlOptimization/ProfitController/default.htm

http://hpsweb.honeywell.com/Cultures/en-US/Products/ControlApplications/Simulation/default.htm”

# 8. Selvoptimaliserende regulering av satsvis destillasjon

Medveileder: Håkon Dahl-Olsen

Oppgaven vil gå ut på å sette opp en dynamisk modell av en satsvis destillasjonsprosess og bruke tilgjengelig teori til å designe et selvoptimaliserende reguleringssystem for denne kolonnen. Ytelsen av reguleringssystemet skal vurderes ved å se på avvik i objektfunksjonen. Et av følgende mål kan velges for driften av prosessen:

1.      Maks produkt med gitt minimumsrenhet ved fast driftstid per batch

2.      Minimum tid for gitt renhet og mengde (absolutt eller gjenvinningsgrad)

Naturlige beregningsverktøy vil være gProms og Matlab. Oppgaven krever interesse for optimalisering og numeriske beregninger. Se hjemmeside S. Skogestad for mer informasjon

## Litteratur

- Sigurd Skogestad og Ian Postlethwaite: Multivariable feedback control – analysis and design (Kap. 10 spesielt), Wiley & Son Ltd, 2005

- Arthur E. Bryson: Dynamic optimization, Addison-Wesley, 1999

- Desineni Subbaram Naidu: Optimal control systems, CRC Press, 2002

9. Dynamic Model and Control of Heat Exchanger Networks for District Heating

Medveileder: Johannes Jäschke

In Trondheim there is a district heating network for hot water based on the Tiller incineration plant for burning waste, and in Oslo there are similar plants. It is possible to save energy by improving the operation and control of such systems. This will be a continuation of a successful ongoing project in cooperation with Helge Mordt at Prediktor (Fredrikstad) who has been working with the Oslo plant.

Depending on the interest of the student, three possible projects are suggested

9.1 Modelling of district heating network

- Literature research on modelling district heating systems for cities

- Develop a simple model

- Combine this model with an existing model of an incineration plant

- Propose a simple control structure and simulate the plant

9.2 Model predictive control of district heating network.

The project is to develop a (possibly non-linear) model predictive controller for a district

heating system.

- Literature study on non-linear MPC and model reduction

- Dynamic model reduction of heat exchangers

- Implementing a non-linear model predictive controller

9.3 Self-optimizing control of polynomial systems

To obtain optimal operation, it is important to identify “self-optimizing” controlled variables. In this project the objective is to find higher-order self-optimizing variables

- Literature study on polynomial systems

- Deriving a polynomial model approximation

- Finding polynomial self-optimizing control variables using elimination techniques

- Implementing the control structure in Simulink

7. "Optimalization of process steps in a production line of contrast agents including charcoal flitration, water and 2- methoxyethanol removal. Increased throughput in the process steps through maximized reactor utilization and optimal regulation".

Several process steps in a production line have to play together to maximize the throughput in the line through regulation of reactor volum, optmal distillation, filtration, concentration and regulation of energy input.

The work will include modelling of the process steps using excisting process parameters and process information.

Concering the secrecy, if we are able to keep the actual names of the product involved out there should not be any problem with that. We need anyway a secrecy agreement with the actual student concerning information we give him/her concerning our process. This will also include the right to exclude information from the master work which we like to be kept secret.

Hilsen Dick

(Didrik Malthe Sørensen)

- 8. Andre oppgaver kan være aktuelle, gjerne i samarbeid med industri eller andre institusjoner