Risikovurderingsrapport

Mini-loop

| Prosjekttittel | 34056204, Master prosjekt: Robust kontroll strategier |
| --- | --- |
| Prosjektleder | Knut Åge Meland |
| Enhet | NTNU |
| HMS-koordinator | Erik Langørgen |
| Linjeleder | Olav Bolland |
| Riggnavn | Mini-Loop |
| Plassering | Faselab |
| Romnummer |  |
| Riggansvarlig | Ole Jørgen Nydal |
| Risikovurdering utført av | Knut Åge Meland og Erik Langørgen |

Innholdsfortegnelse

1	Innledning	1

2	Organisering	1

3	Risikostyring av prosjektet	1

4	Tegninger, foto, beskrivelser av forsøksoppsetT	1

5	Evakuering fra forsøksoppsetningen	1

6	Varsling	2

6.1	Før forsøkskjøring	2

6.2	Ved uønskede hendelser	2

7	Vurdering av teknisk sikkerhet	3

7.1	Fareidentifikasjon, HAZOP	3

7.2	Brannfarlig, reaksjonsfarlig og trykksatt stoff og gass	3

7.3	Trykkpåkjent utstyr	3

7.4	Påvirkning av ytre miljø (utslipp til luft/vann, støy, temperatur, rystelser, lukt).	3

7.5	Stråling	3

7.6	Bruk og behandling av kjemikalier	3

7.7	El sikkerhet (behov for å avvike fra gjeldende forskrifter og normer)	3

8	Vurdering av operasjonell sikkerhet	4

8.1	Prosedyre HAZOP	4

8.2	Drifts og nødstopps prosedyre	4

8.3	Opplæring av operatører	4

8.4	Tekniske modifikasjoner	4

8.5	Personlig verneutstyr	4

8.6	Generelt	4

8.7	Sikkerhetsutrustning	4

8.8	Spesielle tiltak	4

9	Tallfesting av RESTRISIKO – RISIKOMATRISE	4

10	Konklusjon	5

11	Lover Forskrifter og Pålegg som gjelder	6

# Innledning

For å teste forskjellige kontroll strategier ble en små skala J-riser bygget ved institutt for kjemisk prosessteknologi (Bårdsen 2003). Dette eksperimentet ble flyttet til varmeteknisk pga. renovering av K4.

Eksperimentet foregår i korte trekk ut å blande luft og trykkluft som deretter blir transportert opp en riser til en topside separator. Luften blir her frigjort, mens vannet blir resirkulert.

# Organisering

| Rolle | NTNU | Sintef |
| --- | --- | --- |
| Lab Ansvarlig: | Morten Grønli | Harald Mæhlum |
| Linjeleder: | Olav Bolland | Mona J. Mølnvik |
| HMS ansvarlig: HMS koordinator HMS koordinator | Olav Bolland Erik Langørgen Bård Brandåstrø | Mona J. Mølnvik Harald Mæhlum |
| Romansvarlig: |  |  |
| Prosjekt leder: | Knut Åge Meland |  |
| Ansvarlig riggoperatører: | Knut Åge Meland |  |

# Risikostyring av prosjektet

| Hovedaktiviteter risikostyring | Nødvendige tiltak, dokumentasjon | DTG |
| --- | --- | --- |
| Prosjekt initiering | Prosjekt initiering mal |  |
| Veiledningsmøte | Skjema for Veiledningsmøte med pre-risikovurdering | 01.03.11 Kl 09.00 |
| Innledende risikovurdering | Fareidentifikasjon – HAZID Skjema grovanalyse | 08.03.11 Kl 10.00 |
| Vurdering av teknisk sikkerhet | Prosess-HAZOP Tekniske dokumentasjoner | 08.03.11 Kl 10.00 |
| Vurdering av operasjonell sikkerhet | Prosedyre-HAZOP Opplæringsplan for operatører | 08.03.11 Kl 10.00 |
| Sluttvurdering, kvalitetssikring | Uavhengig kontroll Utstedelse av apparaturkort Utstedelse av forsøk pågår kort | 24.03.11 Kl 12:30 |

# Tegninger, foto, beskrivelser av forsøksoppsetT

Vedlegg: Operating manual for the Mini-loop experiment

- Prosess og Instrumenterings Diagram, (PID), se figur 1 i vedlegg.

- Spesifikasjoner om komponenter, se tabeller 1, 2 og 3 i vedlegg.

- Oversiktsbilde over rigg, se figur 2 og 3 i vedlegg.

# Evakuering fra forsøksoppsetningen

Evakuering skjer på signal fra alarmklokker eller lokale gassalarmstasjon med egen lokal varsling med lyd og lys utenfor aktuelle rom, se 6.2

Evakuering fra rigg området foregår igjennom merkede nødutganger.

# Varsling

## Før forsøkskjøring

Varsling per e-post, med opplysning om forsøkskjøringens varighet og involverte til:

- HMS koordinator NTNU/SINTEF

Erik.langorgen@ntnu.no

Baard.brandaastro@ntnu.no

All forsøkskjøringen skal planlegges og legges inn i aktivitetskalender for lab. Forsøksleder må få bekreftelse på at forsøkene er klarert med øvrig labdrift før forsøk kan iverksettes.

## Ved uønskede hendelser

BRANN

Ved brann en ikke selv er i stand til å slukke med rimelige lokalt tilgjengelige slukkemidler, skal nærmeste brannalarm utløses og arealet evakueres raskest mulig. En skal så være tilgjengelig for brannvesen/bygningsvaktmester for å påvise brannsted.

Om mulig varsles så:

| NTNU | SINTEF |
| --- | --- |
| Labsjef Morten Grønli, tlf: 918 97 515 |  |
| HMS: Erik Langørgen, tlf: 91897160 |  |
| Instituttleder: Olav Bolland: 91897209 |  |

GASSALARM

Ved gassalarm skal gassflasker stenges umiddelbart og området ventileres. Klarer man ikke innen rimelig tid å få ned nivået på gasskonsentrasjonen så utløses brannalarm og laben evakueres. Dedikert personell og eller brannvesen sjekker så lekkasjested for å fastslå om det er mulig å tette lekkasje og lufte ut området på en forsvarlig måte.

Varslingsrekkefølge som i overstående punkt.

PERSONSKADE

- Førstehjelpsutstyr i Brann/førstehjelpsstasjoner,

- Rop på hjelp,

- Start livreddende førstehjelp

- Ring 113 hvis det er eller det er tvil om det er alvorlig skade.

ANDRE UØNSKEDE HENDELSER (AVVIK)

NTNU:

Rapporteringsskjema for uønskede hendelser på

http://www.ntnu.no/hms/2007_Nettsider/HMSRV0401_avvik.doc

# Vurdering av teknisk sikkerhet

## Fareidentifikasjon, HAZOP

Forsøksoppsetningen deles inn i følgende noder:

| Node 1 | Water tank and water pump |
| --- | --- |
| Node 2 | Air supply: valves and regulators |
| Node 3 | Air buffer tanks, riser and separator |

Vedlegg, skjema: Hazop_mal

Vurdering: Sikkerhet ivaretatt ved eksisterende og planlagte tiltak beskrevet i vedlegget

## Brannfarlig, reaksjonsfarlig og trykksatt stoff og gass

Inneholder forsøkene brannfarlig, reaksjonsfarlig og trykksatt stoff

| NEI | JA. Eksplosjonsverndokument utarbeides og eller dokumentert trykktest, (kap 7.3) |
| --- | --- |

## Trykkpåkjent utstyr

Inneholder forsøksoppsetningen trykkpåkjent utstyr:

| NEI | JA Utstyret trykktestes i henhold til norm og dokumenteres |
| --- | --- |

Trykkutsatt utstyr skal trykktestes med driftstrykk gange faktor 1.4,for utstyr som har usertifiserte sveiser er faktoren 1.8. Trykktesten skal dokumenteres skriftlig hvor fremgangsmåte framgår.

Vedlegg: Sertifikat for trykkpåkjent utstyr.

Vurdering: Utstyret er testet ved kritisk trykk

## Påvirkning av ytre miljø (utslipp til luft/vann, støy, temperatur, rystelser, lukt).

| NEI | JA |
| --- | --- |

## Stråling

| NEI | JA, Strålekilden skal gjenomgå en egen risikovurdering |
| --- | --- |

## Bruk og behandling av kjemikalier

| NEI | JA, Bruken risikovurderes |
| --- | --- |

Vurdering: Ingen sikkerhetsdatablad er tilgjengelig for fargetilsetningen Vulcanosol Blau. Konsentrasjonen av fargetilsetningen er meget lav, og bruk kan dermed godkjennes ved innførsel av ekstra sikkerhetstiltak. Da denne fargetilsetningen tidligere har blitt bruk i lignende forsøk, anses tilsetningen ikke som ekstremt heleskadelig, men vil likevel behandles som helseskadelig. Det er antatt at risiko ved lekkasjer ikke øker vesentlig, da konsentrasjonen av fargetilsetningen er meget lav samt at trykket i riggen er lavt.

Følgende ekstra sikkerhetstiltak skal innføres ved bruk av fargetilsetningen Vulcanosol Blau:

- Hansker skal benyttes ved behandling av fargetilsetning/ vannløsning.

- Mengde fargetilsetning tilsatt skal være innenfor anbefalt mengde (1 spatel/halv vanntank) gitt i brukermanualen.

- Etter endt forsøksperiode skal rigg og vanntank tømmes. Ingen etterbehandling av avfall ble vurdert som nødvendig, da konsentrasjonen fargetilsetning er lav og mengden løsning er liten.  Avfall tømmes i sluk.

- Andre tiltak: Måle pH til løsningen for å sjekke at løsningen er tilnærmet nøytral.

## El sikkerhet (behov for å avvike fra gjeldende forskrifter og normer)

| NEI | JA, El sikkerhet gjennomgås å risikovurderes |
| --- | --- |

# Vurdering av operasjonell sikkerhet

Sikrer at etablerte prosedyrer dekker alle identifiserte risikoforhold som må håndteres gjennom operasjonelle barrierer og at operatører og teknisk utførende har tilstrekkelig kompetanse.

## Prosedyre HAZOP

Vurdering: Sikkerhet ivaretatt med gjeldende prosedyre

## Drifts og nødstopps prosedyre

- Steng ventilen (V2) som kobler sammen trykkluft kilden med buffertanken.

- Koble fra strømmen til vannpumpen.

## Opplæring av operatører

Vedlegg: Opplæringsplan for operatører

## Tekniske modifikasjoner

Vurdering:

- Tekniske modifikasjoner som ikke endrer risikobilde er tillatt av godkjent operatør

- Ny risikovurdering må utføres om høyere trykk benyttes

## Personlig verneutstyr

Vurdering: Påbudt med vernebriller og egnet skotøy (hansker ved bruk av fargetilsetning).

## Generelt

Vurdering: Sperring settes opp under forsøk

## Sikkerhetsutrustning

Vurdering: Ingen utover utrustning påmontert forsøksoppsett

## Spesielle tiltak

Vurdering: Ingen spesielle tiltak nødvendig

# Tallfesting av RESTRISIKO – RISIKOMATRISE

| IDnr | Aktivitet-hendelse | Frekv-Sans | Kons | RV |
| --- | --- | --- | --- | --- |
| 1 | Lekkasjer under forsøk Lekkasje under forsøk ved bruk av fargetilsetning | 2 2 | A B | A2 B2 |
| 2 | Personer i avsperret område under eksperiment | 2 | A | A2 |
| 3 | For høyt trykk | 2 | B | B2 |
| 4 | Vann i buffertank | 3 | A | A3 |
| 5 | Tørr kjøring av vann pumpe | 2 | A | A2 |
| 6 | Rigg går over flere etasjer: |  |  |  |
|  | 6a Snuble i trapp | 2 | B | B2 |
|  | 6b Lang tid for oppdagelse av en uønsket hendelse | 2 | A | A2 |
| 7 | Skade som følge av bruk av fargetilsetning | 2 | A | A2 |

Vurdering restrisiko: Restrisiko er av akseptabel størrelse ved gjeldende sikkerhetsregler

# Konklusjon

- Riggen er bygget til god laboratorium praksis (GLP).

- Trykktesting bør utføres på nytt om sikkerhetsbilde endres.

- Ny sikkerhetsvurdering må utføres ved store endring av sikkerhetsbilde.

Apparaturkortet får en gyldighet på 6 måneder

Forsøk pågår kort får en gyldighet på 6 måneder

# Lover Forskrifter og Pålegg som gjelder

Se http://www.arbeidstilsynet.no/regelverk/index.html

- Lov om tilsyn med elektriske anlegg og elektrisk utstyr (1929)

- Arbeidsmiljøloven

- Forskrift om systematisk helse-, miljø- og sikkerhetsarbeid (HMS Internkontrollforskrift)

- Forskrift om sikkerhet ved arbeid og drift av elektriske anlegg (FSE 2006)

- Forskrift om elektriske forsyningsanlegg (FEF 2006)

- Forskrift om utstyr og sikkerhetssystem til bruk i eksplosjonsfarlig område NEK 420

- Forskrift om håndtering av brannfarlig, reaksjonsfarlig og trykksatt stoff samt utstyr og anlegg som benyttes ved håndteringen

- Forskrift om Håndtering av eksplosjonsfarlig stoff

- Forskrift om bruk av arbeidsutstyr.

- Forskrift om Arbeidsplasser og arbeidslokaler

- Forskrift om Bruk av personlig verneutstyr på arbeidsplassen

- Forskrift om Helse og sikkerhet i eksplosjonsfarlige atmosfærer

- Forskrift om Høytrykksspyling

- Forskrift om Maskiner

- Forskrift om Sikkerhetsskilting og signalgivning på arbeidsplassen

- Forskrift om Stillaser, stiger og arbeid på tak m.m.

- Forskrift om Sveising, termisk skjæring, termisk sprøyting, kullbuemeisling, lodding og sliping (varmt arbeid)

- Forskrift om Tekniske innretninger

- Forskrift om Tungt og ensformig arbeid

- Forskrift om Vern mot eksponering for kjemikalier på arbeidsplassen (Kjemikalieforskriften)

- Forskrift om Vern mot kunstig optisk stråling på arbeidsplassen

- Forskrift om Vern mot mekaniske vibrasjoner

- Forskrift om Vern mot støy på arbeidsplassen

Veiledninger fra arbeidstilsynet

se: http://www.arbeidstilsynet.no/regelverk/veiledninger.html

Vedlegg til

Risikovurderingsrapport

Mini-Loop

| Prosjekttittel | 34056204, Master prosjekt: Robust kontroll strategier |
| --- | --- |
| Prosjektleder | Knut Åge Meland |
| Enhet | NTNU |
| HMS-koordinator | Erik Langørgen |
| Linjeleder |  |
| Riggnavn | Mini-Loop |
| Plassering | Faselab |
| Romnummer |  |
| Riggansvarlig |  |

Innholdsfortegnelse

	Vedlegg A HAZOP MAL	1

	Vedlegg B Prøvesertifikat for lokal trykktesting	1

	Vedlegg G Forsøksprosedyre	1

	Vedlegg H Opplæringsplan for opperatører	2

	Vedlegg K Forsøk pågår kort	3

# Vedlegg A HAZOP MAL

| Project: Master project: Robust control strategies Node: 1, water supply |  |  |  |  |  |  | Page |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref # | Guideword | Causes | Consequences | Safeguards | Recommendations | Action |  | Date Sign |
| 1.1 | No flow | Air in pump or low water level in water tank | Pump could be damaged | Follow startup and shutdown procedures |  |  |  |  |
| 1.2 | Reverse flow | High air pressure | Water splash from water tank or damaged pump if run dry | Follow startup and shutdown procedures. Establish a safety zone when running the experiment |  |  |  |  |
| 1.3 | More flow | Dependent on the V3 valve position | unknown |  | Test if the equipment could handle maximum water flow | The equipment was tested with maximum waterflow |  | 22.03.2011 Knut Åge Meland |
| 4 | Less flow | Ref 1.1 Ref 1.3 |  |  |  |  |  |  |
| 5 | More level | Too much water in tank | Water splash/leakage from tank | Check level before startup and during experiment. Establish a safety zone when running the experiment |  |  |  |  |
| 6 | Less level | Too little water in tank | Water pump could run dry and potential damage could occur | Check level before startup and during experiment. |  |  |  |  |
| 7 | More pressure | Ref 1.3 |  |  |  |  |  |  |
| 8 | Less pressure | Ref 1.4 |  |  |  |  |  |  |
| 9 | More temperature | Not relevant |  |  |  |  |  |  |
| 10 | Less temperature | Not relevant |  |  |  |  |  |  |
| 11 | More viscosity | Not relevant |  |  |  |  |  |  |
| 12 | Less viscosity | Not relevant |  |  |  |  |  |  |
| 13 | Composition Change | Not relevant |  |  |  |  |  |  |
| 14 | Contamination | Growth of algae in the tank caused by stationary water | Could clog equipment or pipe Ref 1.1 | Empty tank if the experiment is not planned to be used in the near future | Put inside procedure | Added to the procedure |  | 22.03.2011 Knut Åge Meland |
| 15 | Relief | Not relevant |  |  |  |  |  |  |
| 16 | Instrumentation | Not relevant |  |  |  |  |  |  |
| 17 | Sampling | Not relevant |  |  |  |  |  |  |
| 18 | Corrosion/erosion | Corrosion could occur if water is trapped in the system for long periods of time | Equipment failure/clogging | Remove all water if the experiment is not to be in use in the near future. | Put inside procedure | Added to the procedure |  | 22.03.2011 Knut Åge Meland |
| 19 | Service failure | Not relevant |  |  |  |  |  |  |
| 20 | Abnormal operation | Not relevant |  |  |  |  |  |  |
| 21 | Maintenance | Not relevant |  |  |  |  |  |  |
| 22 | Ignition | Not relevant |  |  |  |  |  |  |
| 23 | Spare equipment | Not relevant |  |  |  |  |  |  |
| 24 | Safety | Not relevant |  |  |  |  |  |  |

| Project: Master project: Robust control strategies Node: 2, air supply: valves and regulators |  |  |  |  |  |  | Page |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref # | Guideword | Causes | Consequences | Safeguards | Recommendations | Action |  | Date Sign |
| 2.1 | No flow | Closed valves (V1, V2) or pressure regulator. Air supply is out of order | Water in buffertank | Follow procedure |  |  |  |  |
| 2.2 | Reverse flow | Not possible |  |  |  |  |  |  |
| 2.3 | More flow | Dependent on the V2 valve position | None |  |  |  |  |  |
| 2.4 | Less flow | Ref 2.3 Out of order air supply | Water in buffertank | The connection from buffer tank to mixing point is elevated. |  |  |  |  |
| 2.5 | More level | Not relevant |  |  |  |  |  |  |
| 2.6 | Less level | Not relevant |  |  |  |  |  |  |
| 2.7 | More pressure | Position of the pressure regulator | Explosion air buffer tank or leaks | 1 barg safety valve installed Establish a safety zone when running the experiment. Safety glasses | Do a pressure test before each period of experiments to ensure operational safety valve | A pressure test of the rig was completed and test procedure was included into the operation manual |  | 22.03.2011 Knut Åge Meland |
| 2.8 | Less pressure | Position of the pressure regulator. Leaks | Water can flow into the air buffer tank. | The connection from buffer tank to mixing point is elevated. |  |  |  |  |
| 2.9 | More temperature | Not relevant |  |  |  |  |  |  |
| 2.10 | Less temperature | Not relevant |  |  |  |  |  |  |
| 2.11 | More viscosity | Not relevant |  |  |  |  |  |  |
| 2.12 | Less viscosity | Not relevant |  |  |  |  |  |  |
| 2.13 | Composition Change | Not relevant |  |  |  |  |  |  |
| 2.14 | Contamination | Not relevant |  |  |  |  |  |  |
| 2.15 | Relief | Not relevant |  |  |  |  |  |  |
| 2.16 | Instrumentation | Not relevant |  |  |  |  |  |  |
| 2.17 | Sampling | Not relevant |  |  |  |  |  |  |
| 2.18 | Corrosion/erosion | Not relevant |  |  |  |  |  |  |
| 2.19 | Service failure | Not relevant |  |  |  |  |  |  |
| 2.20 | Abnormal operation | Not relevant |  |  |  |  |  |  |
| 2.21 | Maintenance | Not relevant |  |  |  |  |  |  |
| 2.22 | Ignition | Not relevant |  |  |  |  |  |  |
| 2.23 | Spare equipment | Not relevant |  |  |  |  |  |  |
| 2.24 | Safety | Not relevant |  |  |  |  |  |  |

| Project: Master project: Robust control strategies Node: 3, air buffer tanks, riser and separator |  |  |  |  |  |  | Page |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref # | Guideword | Causes | Consequences | Safeguards | Recommendations | Action |  | Date Sign |
| 3.1 | No flow | Closed valves or choke valve Clogged water recycle | Leaks Water in buffertank | Do pressure tests Follow procedure Establish a safety zone when running the experiment. Use safety glasses |  |  |  |  |
| 3.2 | Reverse flow | Ref 3.1 |  |  |  |  |  |  |
| 3.3 | More flow | Dependent on choke valve position or valve (V2). Ref 1.3 | Water splash from leaks or the separator air outlet | A piece of pipe is connected to the outlet Establish a safety zone when running the experiment. Use safety glasses |  |  |  |  |
| 3.4 | Less flow | Blown safety valve or leaks | Water spill | Establish a safety zone when running the experiment. |  |  |  |  |
| 3.5 | More level | More flow to the separator. Clogged water recycle | Water splash from the separator air outlet | A piece of pipe is connected to the outlet |  |  |  |  |
| 3.6 | Less level | Less flow to the separator | None |  |  |  |  |  |
| 3.7 | More pressure | Position of the choke valve Ref 2.7 | Explosion of one of the air buffer tank. Leaks | 1 barg safety valve installed. Establish a safety zone when running the experiment. Use safety glasses. |  |  |  |  |
| 3.8 | Less pressure | Position of the pressure regulator and choke valve | Water can flow into the air buffer tank. | The connection from buffer tank to mixing point is elevated. |  |  |  |  |
| 3.9 | More temperature | Not relevant |  |  |  |  |  |  |
| 3.10 | Less temperature | Not relevant |  |  |  |  |  |  |
| 3.11 | More viscosity | Not relevant |  |  |  |  |  |  |
| 3.12 | Less viscosity | Not relevant |  |  |  |  |  |  |
| 3.13 | Composition Change | Change in pressure or flow rates of water and air | May change flow regime in the riser |  |  |  |  |  |
| 3.14 | Contamination Ref 1.14 |  | Leaks Pressure build up |  |  |  |  |  |
| 3.15 | Relief | Not relevant |  |  |  |  |  |  |
| 3.16 | Instrumentation | Not relevant |  |  |  |  |  |  |
| 3.17 | Sampling | Not relevant |  |  |  |  |  |  |
| 3.18 | Corrosion/erosion Ref 1.18 |  |  |  |  |  |  |  |
| 3.19 | Service failure | If the safety valve testing is neglected | Explosion of the buffertank | Follow procedure Proper training Establish a safety zone when running the experiment. Use safety glasses. |  |  |  |  |
| 3.20 | Abnormal operation | Not relevant |  |  |  |  |  |  |
| 3.21 | Maintenance | Not relevant |  |  |  |  |  |  |
| 3.22 | Ignition | Not relevant |  |  |  |  |  |  |
| 3.23 | Spare equipment | Not relevant |  |  |  |  |  |  |
| 3.24 | Safety | Not relevant |  |  |  |  |  |  |

# Vedlegg B Prøvesertifikat for lokal trykktesting

Trykktesten skal utføres I følge NS-EN 13445 del 5 (Inspeksjon og prøving).

Se også prosedyre for trykktesting gjeldende for VATL lab

Trykkpåkjent utstyr:	Air buffer tanks

Benyttes i rigg:	Mini-loop

Design trykk for utstyr:				0.1-0.5 bara

Maksimum tillatt trykk:					1 bara

(i.e. burst pressure om kjent)

Maksimum driftstrykk i denne rigg:			0.5 bara

Prøvetrykket skal fastlegges i følge standarden og med hensyn til maksimum tillatt trykk.

Prøvetrykk:_______ bara 	(1.4 x maksimum driftstrykk)

I følge standard

Test medium:

Temperatur: 			      °C

Start:		Tid:

Trykk: 				bara

Slutt: 		Tid:

Trykk: 				bara

Eventuelle repetisjoner fra atm. trykk til maksimum prøvetrykk:…………….

Test trykket, dato for testing og maksimum tillatt driftstrykk skal markers på (skilt eller innslått)

Sted og dato							Signatur

# Vedlegg G Forsøksprosedyre

| Experiment, name, number: Robust control strategies | Date/ Sign |
| --- | --- |
| Project Leader: Ole Jørgen Nydal |  |
| Experiment Leader: Knut Åge Meland |  |
| Operator, Duties: Knut Åge Meland |  |

|  | Conditions for the experiment: | Completed |
| --- | --- | --- |
|  | Experiments should be run in normal working hours, 08:00-16:00 during winter time and 08.00-15.00 during summer time. Experiments outside normal working hours shall be approved. |  |
|  | One person must always be present while running experiments, and should be approved as an experimental leader. |  |
|  | An early warning is given according to the lab rules, and accepted by authorized personnel. |  |
|  | Be sure that everyone taking part of the experiment is wearing the necessary protecting equipment and is aware of the shut down procedure and escape routes. |  |
|  | Make sure that the water level is sufficiently high, no apparent leaks and no contamination of the water (algae growth, corrosion). |  |
|  | Preparations | Carried out |
|  | Post the “Experiment in progress” sign. |  |
|  | Perform safety valve test before each period of operation (see operating manual page 4) |  |
|  | Start up procedure (see operating manual page 4) |  |
|  | During the experiment |  |
|  | Control of water level |  |
|  | Look for leaks/spills |  |
|  | End of experiment |  |
|  | Shut down procedure (see operating manual page 4) |  |
|  | Empty the water tank and remove water from the pipes (only necessary for long shut down periods). |  |
|  | Remove all obstructions/barriers/signs around the experiment. |  |
|  | Tidy and cleanup work areas. |  |
|  | To reflect on before the next experiment and experience useful for others |  |
|  | Was the experiment completed as planned and on scheduled in professional terms? |  |
|  | Was the competence which was needed for security and completion of the experiment available to you? |  |
|  | Do you have any information/ knowledge from the experiment that you should document and share with fellow colleagues? |  |

# Vedlegg H Opplæringsplan for opperatører

| Experiment, name, number: Robust control strategies | Date/ Sign |
| --- | --- |
| Project Leader: Ole Jørgen Nydal |  |
| Experiment Leader: Knut Åge Meland |  |
| Operator Knut Åge Meland |  |

|  | Kjennskap til EPT LAB generelt |  |
| --- | --- | --- |
|  | Lab - adgang -rutiner/regler -arbeidstid |  |
|  | Kjenner til evakueringsprosedyrer |  |
|  | Aktivitetskalender |  |
|  | Kjennskap til forsøkene |  |
|  | Prosedyrer for forsøkene |  |
|  | Nødstopp |  |
|  | Nærmeste brann/førstehjelpsstasjon |  |
|  | Hvor alle ventiler og annet utstyr er plassert |  |
|  | Kjennskap til fargetilsetningen, Vulcanosol Blau, og skal behandles som om det er helseskadelig. |  |

# Vedlegg K Forsøk pågår kort

Forsøk pågår!

Experiment in progress!

Dette kort skal settes opp før forsøk kan påbegynnes This card has to be posted before an experiment can start

| Ansvarlig / Responsible Knut Åge Meland | Telefon jobb/mobil/hjemme 91104536 |
| --- | --- |
| Operatører/Operators Knut Åge Meland | Forsøksperiode/Experiment time(start – slutt) |
| Prosjektleder Ole Jørgen Nydal | Prosjekt Master project, Robust control strategies |
| Kort beskrivelse av forsøket og relaterte farer Short description of the experiment and related hazards Water from the water tank and pressurized air from the internal laboratory supply are mixed together to form a two phase flow. The two phase flow is driven by pressure forces through the riser into a topside separator where the liquid and air are separated. The water is recycled to the water storage tank and the low pressurized air is released into the environment. Hazards related to the experiment involve water leakage, water spills, blow-out of low pressurized tank. |  |

| NTNU Institutt for energi og prosessteknikk |  | SINTEF Energi Avdeling energiprosesser |
| --- | --- | --- |
| Dato |  | Dato |
| Signert |  | Signert |

Operating manual for the Mini-loop experiment

Updated: March 2011by Knut Åge Meland

Contents

Introduction	6

Test and calibration of safety valve	7

Startup procedure	7

Shutdown procedure	7

Comments	7

Adjusting flow rates	8

Emergency shutdown procedure	8

Overview of the experimental setup	9

Reference	13

Appendix A	14

# Introduction

This manual contains a rewrite of the startup and shutdown procedure found in the user manual found the thesis of Sondrol [1].  The old manual is given in appendix A and contains more detailed information about the LabVIEW program. However, some small modifications to the programs have been done. It must be noted that the startup and shutdown procedure given in the manual of Sondrol should not be used. The experimental setup has also since described by Sondrol undergone some modification and will not completely correspond to the drawings given in the manual by Sondrol.

# Test of safety valve

- Make sure that the chokevalve and V3 is closed.

- Open valve V1 for pressure.

- Adjust the pressure regulator such that the pressure is slightly above 1 bar.

- Open the valve V2

- Safety valve has been activated when air is leaking from it. Listen for leaks, and monitor pressure.

# Startup procedure

- Make sure all manual valves are closed and there is no visible leaks/spills

- Start the computer and open LabVIEW.

- Connect the power to the field point modules.

- Turn the field point modules on by using the switch located inside the control cabinet.

- Put the Mini-loop program into run mode and open chokevalve fully.

- Connect the power to the pump.

- Open the master air flow valve (V1) located beside the door to the workshop

- Adjust the pressure using the pressure regulator (usually not needed)

- Open valve V2 until the desired air flow is reached.

- Turn valve V3 until the desired water flow is reached

# Shutdown procedure

- Shut of the water supply first (valve V3 first), and then the air supply (valve V2)

- Disconnect the power supply to the pump.

- Close valve V1 and disconnect the hose from the valve

- Turn of the field point modules with the switch inside the cabinet.

- Disconnect the power source to the field point modules.

- Close LabView and shut down the computer.

Note: For long shut down periods also remove water from the pipes and empty the water tank.

# Comments

- The air supply must always be turned on first and shut down last. The reason is to obtain a certain pressure inside the hose to prevent backflow of water into the buffer tank (T1).

- The pump will start to work as soon as the power supply is connected. So make sure there is no air in the pipe leading from the water reservoir to the pump. Also make sure that the water level in the reservoir tank is higher than the outlet leading to the pump.

- The valve will always close itself when the field point modules are turned off. The mini-loop program therefore has to be put in run mode to open the valve before air or water is introduced to the system. Failure to do this will result in a quick pressure build up in the pipe and a blow out of the pressure sensors.

# Adjusting flow rates

During system start up it is recommended to adjust the air flow to the desired flow rate before introducing water into the system. Once the air flow is adjusted the water flow rate can be adjusted to the desired rate. Take note that the water flow rate will vary depending on the upstream pressure. The water flow will normally vary around 10%. To maintain consistency it’s recommended to use the max flow rate during these variations as the reference. The air flow measurement is dependent on the temperature inside the measuring device. This means that the measured flow rate of air will drop during the first minutes after start up as the temperature inside the device stabilize itself. Because of this, it’s not recommended to initiate any experiments until the measurement is stable (approx.: 5-10 mins.)

# Emergency shutdown procedure

- Close the air valve (V2) connecting the air supply to the buffer tanks

- Disconnect the power to the water pump

# Overview of the experimental setup

Table 1 Equipment list

| Equipment List |  |  |  |  |
| --- | --- | --- | --- | --- |
| Displayed Text | Description | Manufacturer | Material | Model |
| Pump | Water pump | Grundfos |  | Solarpumpe UPS 25-120-180 |
| T1 | Air buffer tank | NTNU | Plexiglass |  |
| T2 | Separator | NTNU | Plexiglass |  |
| T3 | Water storage tank | NTNU | Plexiglass |  |

Table 2 Instrument list

| Instrument List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displayed Text | Description | Connection Size | Service | Manufacturer | Model |
| FT-A | Flowmeter for the air supply |  | Max pressure 150 psi | Coleparmer | 32707-36 |
| FT-W | liquid flow transmitter |  |  | Gemü | 3021-25-D-7-1-4-A-T41-C1 |
| P1 | Pressure transmitter |  |  | Motorola | MPX 1500DP |
| P2 | Pressure transmitter |  |  | Motorola | MPX 1500DP |
| P3 | Pressure transmitter |  |  | Motorola | MPX 1500DP |

Table 3 Valve list

| Valve List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displayed Text | Description | Line Size | Valve Class | Manufacturer | Model |
| CV | Choke valve | max operating pressure 20 bar. Min. operatin pressure 5.5-7.0 bar | Angle seat valve | Gemü | 554 20D 195 2 1 |
| Pressure regulator | Pressure regulator for air |  |  | Mecman | 915-112-044-1 |
| Safety valve | Safety valve for preventing rapture of the buffertank |  | 1 barg | Flamco | Prescor |
| V1 | Master valve for air supply |  |  |  |  |
| V2 | Valve controlling the flowrate of air |  |  |  |  |
| V3 | Globe valve for controlling water flowrate |  |  |  |  |

Figure 1 P&ID diagram of the experiment

Figure 2 Overview of the experiment.

Figure 3 Location of valve V1

# Reference

[1] Sondrol, M (2005), Anti-slug control. Experimental testing and veriﬁcation, Master Thesis at Department of Chemical Engineering NTNU, Trondheim, 112.

# Appendix A

Old user manual by Sondrol