Optimisations with Energy Recovery for Oxidative Desulphurization of Heavy Gas Oil

Hamza A. Khalfalla a, Iqbal M. Mujtaba.a, Mohamed M. El-Garnib, Hadi A. El-Akramic

aSchool of Engineering Design & Technology, University of Bradford

Bradford BD7 1DP, UK.b

Libyan Petroleum Institute, P.O. Box 6431 Tripoli-Libya

cChemical Engineering Al-Fateh University P.O. Box 13335 Tripoli Libya

Abstract

A lab-scale batch reactor is used to study deep desulphurization of a model sulfur compound dibenzothiophene (DBT) and heavy gas oil (HGO) with hydrogen peroxide (H2O2) as oxidant and formic acid (HCOOH) as catalyst. The results are quite promising and therefore a large scale oxidation process using a continuous stirrer tank reactor (CSTR) is considered further. In this process, a large amount of energy is required to carry out reaction at temperature close to that of the batch reactor, the recovery of which is very important for maximizing the profitability of operation and reducing environmental impact. Therefore a heat integrated CSTR system is proposed here. In the absence of a real plant a model for the system is developed. The kinetic model for the CSTR is based on the batch reactor experiments. An optimization problem to minimize the overall annual plant cost is formulated and solved using gPROMS. A cost saving of 22% for the integrated process is obtained compared to a non-integrated process.

Keywords: Desulfurization, Heavy Gas Oil, Integrated Process, Energy Recovery.

  - Introduction

Both economics and environment dictates the removal of sulfur from petroleum or its products. Sulfur in petroleum products poisons catalytic converters, corrodes parts of internal combustion engines and refineries because of the formation of oxy-acids of sulfur. Traditionally catalytic hydrodesulfurization (HDS) method is used for reducing sulfur which requires high partial pressure of hydrogen and high temperature. This makes HDS a costly option for deep desulfurization. Furthermore HDS is not effective for removing hetrocycilc sulfur compounds such as dibenzothiophene (DBT) and its derivatives. Faced with continuing fuel quality challenges, refiners have begun to look at oxidative desulfurization (ODS), under much milder conditions, as an alternative complementary process to HDS for deep desulfurization (Aida et. al., 2000).

The ODS is basically a two-stage process, oxidation, followed by liquid extraction. In the oxidation step, the sulfur containing compounds are oxidized using appropriate oxidants to convert these compounds to their corresponding sulphones. These are preferentially extracted from oil based on their increased relative polarity (Babich and Moulijn, 2003, Gore; 2001). In the extraction step, the oxidized compounds are extracted from the oil by using a non-miscible solvent.

With this back drop, the aim of this work is two fold. Firstly, the oxidation of model sulfur compound (DBT) and sulfur present in heavy gas oil (HGO) with H2O2 in the presence of a catalyst (HCOOH) is studied. A series of batch experiments are carried out using a small reactor (500 ml) operating at various temperatures ranging from 40 0C to 100 0C. Kinetic model for the oxidation is also developed based on the experimental data. Secondly, a CSTR model is developed for the oxidation process for evaluating the viability of a large-scale operation. Note that while the energy consumption and recovery issues could be ignored for batch experiments those are certainly not the case for large scale operation. To carry out the reaction even at 30-40 0C, the large scale operation will demand large amount of heating. The recovery of heat therefore, is very important for maximizing the profitability of operation. For industrial operation, the calculation of the minimum heating and cooling requirements reveal significant energy savings (Douglas, 1988). Therefore an integrated process where most of the energy is recovered is proposed here. However, this leads to embedding a number of heat exchangers in the system requiring capital investment. Therefore an optimization problem is formulated to minimise the total cost while optimising several design and operating parameters such as reaction temperature, residence time, minimum approach temperatures and splitter ratio. Here, the modelling and optimization are carried out by using gPROMS software (2005).

  - Batch Reactor Experimentation with Small Scale Oxidation Process

The oxidation reaction was carried out in a 500 ml four necked flask containing 30 ml of model oil (sulfur was 940 ppm) and 1.25 ml of 30 % H2O2 (oxidant). Dibenzothiophene (DBT) was dissolved in dodecane to make the model oil. The flask is placed into the heating mantel equipped with a temperature controller and stirred at 750 rpm. When the required reaction temperature has been reached 30 ml of HCOOH (catalyst) was added to the flask to initiate the reaction. This procedure was carried at different temperatures (40, 60, 80 and 100 0C) for different time intervals. The resulting mixture was cooled to room temperature, and the sulfur content of the organic layer was detected by X-ray fluorescence. The same procedure was used for the heavy gas oil (HGO, sulfur was 1066 ppm) using equal volumes of the catalyst and HGO (30 ml) and half the volume of oxidant (15 ml).

    - Experimental results

The results of oxidation of dibenzothiophene with H2O2 as a function of reaction time and reaction temperatures are shown in Figure 1 (Khalfalla et al, 2007). The initial reaction rate was found to be high, therefore, ninety eight (98) percent (wt %) conversion of DBT has been achieved after 5 minutes for all reaction temperatures. The oxidation of HGO with H2O2 as a function of reaction time over various temperatures is shown in Figure 2. The results indicated that the oxidation activities increased with the increasing temperature up to 600C but the conversion is only 40 %. A linear relationship of ln(CA /CA0) versus time was obtained for DBT and total sulfur in HGO for all temperatures (Figures 3 and 4 shows the plot for 40 0C). These results suggest that the oxidative reaction can be treated as a first-order reaction. Therefore, the reaction rate constants at various temperatures can be obtained from the slops of ln(CA /CA0) vs time. The apparent activation energies (E) and Arrhenius factor (A) with model oil and HGO were obtained from the Arrhenius plots (ln K vs 1/T). For HGO, E =7622 J/mol, A= 0.2279 min-1 are obtained. (Khalfalla et. al., 2007)

Figure 1. Oxidation of DBT at different     	Figure 2. Oxidation of HGO at different

reaction temperatures                      	reaction temperatures

Figure 3. First-order plots DBT at 40 0C        	      Figure 4. First-order plots of HGO at 40 0C

  - Heat Integrated Large Scale Oxidation Process

For large scale oxidation a CSTR is chosen. Batch reactor experiment (section 2) shows that oxidation reaction of model sulfur compound and total sulfur in heavy gas oil is favourable at higher temperature (>40 0C). The energy consumption for batch reactor (lab-scale) was negligible (as the volume is small) and natural cooling after the reaction was sufficient and therefore, no additional utility was required. However, in the large scale operation even to raise the HGO at 40 0C, energy consumption is a big issue. Therefore while scaling up a heat integrated of CSTR process (Fig. 5) is considered to reduce the overall energy consumption. The objective is to determine a retrofit design that can reduce the energy consumption, maximize energy recovery and minimize the capital investment. The exchangers, heaters and cooler are represented in Figure 5 by E, H and C respectively. The feed and product temperatures will be considered fixed and equal (TF0). For simplicity a simple CSTR model with the assumption of perfect mixing is used.

    - Process model

In Figure 5, three feed streams were fed into the reactor. The first stream S1 (cold stream) containing HGO is preheated from TF0 to TF1 in the heat exchanger E1. Then, the HGO is fed into the heater H1 to preheat from TF1 to the reaction temperature (Tr). The second stream S2 (cold stream) containing HCOOH is preheated from TF0 to TF2 in the heat exchanger E2, and then from TF2 to Tr through the heater H2. The third stream S3 containing oxidant (H2O2) is fed into heat exchanger E3 to preheat from TF0 to TF3 and then to heat up to the reaction temperature (Tr) in the heater H3. The stream leaving the reactor (hot stream) is divided into two streams (S5 and S6) according to the splitter ratio (Sr). Stream S5 is cooled from Tr to T0 by exchanging heat with the feed stream S1 through the heat exchanger E1. Stream S6 is cooled from Tr to T01 by stream S2 in the heat exchanger E2. The product streams (S5 and S6) are mixed and cooled from TP to TA by exchanging heat with feed stream S3 through the heat exchanger E3, and then from TA to product temperature TF0 (same as feed temperature) in the cooler (C) by using utility water at Tw1.The model equations for the whole process are shown in Figure 6. There are forty six equations and fifty seven variables (fifty three unknown and four specified) and nineteen fixed parameters (shown in Table 1) in these equations. TF0, Tw1, Ts and V0 are specified and ΔT2, ΔT4, ΔT6, Tr, Tw2, τ and Sr are relaxed and optimized.

Figure 5. Process flowsheet of heat - integrated reaction system

  - Optimization Problem Formulation

The optimization problem can be described as follows:

Given	feed and product temperature (TF0), steam temperature (Ts), water temperature (Tw1) and volumetric flow rate of feed (V0);

Optimize	residence time (τ), reaction temperature (Tr), outlet temperature of cooled water (Tw2), splitter ratio (Sr), minimum approach temperatures (ΔT2, ΔT4, ΔT6)

So as to

Minimize	the total cost of the process (Ct) including capital investment

Subject to	constraints on the conversion (0.40<XA<0.42) and linear bounds on all optimization variables

The objective function in the optimization problem formulation is the overall annual plant cost (Ct) that considers equipment cost (reactor Cd, heat exchangers CE, pumping CPU), and operating cost (C0) (Fernando and Pedro, 1998) (Equation 1). The correlations of Cd, CE, CPU and C0 are shown in Figure 7. The optimization problem is solved using gPROMS (2005).

Ct=Cd+CE+C0+CPU						(1)

  - Optimization Results

The results of optimization problem (summarized in Table 2) show that, the minimum total cost (Ct), amounts of steam and cooling water with heat integration of the oxidation process are less than those without the heat integration at specified variables (TF0 = 300 K, Tw1 = 298.15 K, Ts = 373.15 K and V0 = 0.1 m3/min). Furthermore the energy input without heat integration is 126% higher than that with heat integration of the oxidation process. Note the minimum approach temperatures for the heat exchangers E1, E2 and E3 (ΔT2, ΔT4 and ΔT6) are within 19-30 0K which is quite practical.

Q1E1=(V0ρHCPH)(Tr−ΔT1−TF0),ΔT1=Tr−TF1,ΔT2=T0−TF0

Q2E1=Sr(V0ρHCPH+V0ρFCPE+V0H2ρH2CPH2) (Tr−(TF0+ΔT2)),Q1E1=Q2E1

Q1E2=(1−Sr)(V0ρHCPH+V0ρFCPE+V0H2ρH2CPH2) (Tr−(TF0−ΔT)), ΔT4=T01−TF0

Q2E2=V0FρFCPF(Tr−ΔT3−TF0), Q1E2=Q2E2ΔT3=Tr−TF2,

Q1E3=(V0ρHCPH+V0ρFCPE+V0H2ρH2CPH2) (TP−(TF0+ΔT6)), ΔT5=TP−TF3

Q2E3=V0H2ρH2CPH2(TP−ΔT5−TF0), Q1E2=Q2E2,ΔT6=TA−TF0

AE1=Q1E1UEΔTlm1,AE2=Q1E2UEΔTlm2,AE3=Q1E3UEΔTlm3

QH1=V0ρHCPHΔT1 ,QH2=(V0FρFCPF)ΔT3 , Q1H3=V0H2ρH2CPH2(TF3−TF0)

QS1=MS1λ , QH1=QS1   ,QS2=QH2 , QS2=MS2λ

Q2H3=(V0ρHhCPHh+V0FρFhCPFh+V0H2ρH2CPH2) (TP−TA)

AH1=QH1UHΔTlmH1 ,      AH2=QH2UHΔTlmH2   ,         AH3=QH3UHΔTlmH3

QC=(V0ρHCPH+V0FρFCPH+V0H2ρH2CPH2)(TP−TF0), QC=Mwcpw(Tw2−Tw1)

AC=QCUCΔTlmC  ,TP=(SrT0+(1−Sr)T01)

XA=−rAτCA0   ,  FA0=ν CA0   ,  CA=CA0(1−XA) , −rA=A e−ERTCA

Figure 6.Model equations of heat - integrated reaction system

Table 1. Nominal and values of constant parameters used in the model

| Initial concentration, CA0, mol/ m3= 33.3 | Activation energy, E, J/mol = 7622 |
| --- | --- |
| Arrhenius Factor, A, min-1= 0.2279 | Heat capacity of H2O2, CPH2, J/kg K = 3517 |
| Density of H2O2, ρH2 ,kg/m3 = 1400 | Heat capacity of HGO, CPH, J/kg = 1988.73 |
| Heat capacity of HCOOH, CPF ,J/kg = 1730 | Heat capacity of water, CPw, J/kg = 4181.3 |
| Density of HGO, ρH, kg/m3 = 817 | Density of HCOOH, ρF, kg/m3 = 1220 |
| Gas constant, R, J/mol K = 8.314 | Latent heat, λ, J/kg = 2256918 |
| Over all heat transfer coefficient for exchanger UE1, UE2 , UE3, W/m2K = 321 |  |
| Over all heat transfer coefficient for heater, UH1, UH2, , UH3,W/m2K = 851 |  |
| Over all heat transfer coefficient for cooler, UC, W/m2K = 638 |  |

Table 2. Results of optimisation problem

|  | With heat integration | Without heat integration | Decision variable type | Optimized value |
| --- | --- | --- | --- | --- |
| A, m2 | 86.11 | 66.15 | ΔT2, K | 27.1613 |
| Ct,$ | 65887.8 | 84454.7 | ΔT4, K | 30.842 |
| CS*, % | 22 | 0.0 | ΔT6, K | 19.000 |
| MS, kg/min | 4.30 | 12.47 | Sr, % | 0.5932 |
| MW, kg/min | 334 | 356.46 | , minτ | 44 |
| MCR, kJ | 9770 | 2.21E04 | Tr, K | 343.5 |
| MER , kJ | 9770 | 2.21E04 | Tw2, K | 305.15 |
| MHR , kJ | 1.26E04 | 0.0 | XA | 0.410 |
| ES, % | 57 | 0.0 | ------ | ----- |

*CS = Cost saving, MCR= Minimum cooling requirement, ES= Energy saving, MER= Minimum energy requirement, MHR= Maximum heat recovery

Figure 7. Correlation for different cost functions.

Cd,$/yr=4.18(900280)(937.73)(Vπ)0.6227log10CE=K1+K2log10A+K3(log10A)2C0,$/yr900760(7896−6327 q+47640 q2−10220 q4)Cpu,$/yr(900834)(38.63)(264.2FA0)0.8050

Where: K1 = 4.8306, K2 = -0.8596, K3 = 0.3187, A is the total heat exchanger surface area, m2, V is the reactor volume, m3 and q= (QH+QC) /QN and QN=2.54*107 J/min.

  - Conclusions

This study investigates the removal of DBT from model oil and total sulfur from heavy gas oil by oxidative desulfurization in a small batch reactor with hydrogen peroxide as the oxidant in the presence of formic acid as the catalyst. The effectiveness of sulfur removal is found to be proportional to the reaction temperature in the range of 40 to 60 ºC. The sulfur removal of DBT-containing model oil can reach 40-98% under different reaction temperatures. A heat integrated CSTR system is considered to combat high energy demand for a large scale oxidation process. Using optimization techniques within gPROMS, minimum energy input, maximum heat recovery and minimum investment and operating cost are determined. The results show that the cost and energy savings are in the range of 22% and 57% respectively which are significant.

References

T. Aida, D. Yamamoto, M. Iwata and K. Sakata, 2000, Heteroatom Chem., 22, 241.

I. Babich, and J. Moulijn., 2003, Fuel, 82, 607.

J. Douglas., 1988, Conceptual Design of Chemical Processes, McGraw-Hill.

P. Fernando and M. Pedro, 1998, AIChE J., 44, 2007.

W. Gore, USA Patents 6,160,193, 12. 12. 2000, 6,274,785, 14. 08. 2001

H. A. Khalfalla , I. M. Mujtaba., M. El-Garni and H. El-Akrami, 2007, Chemical Engineering Transactions, 11, 53.

gPROMS, (2005), Introductory user guide, Process System Enterprise Ltd (PSE)

http://www. psenterprise.com/gproms/.