# Minimum peaks for some closed loop transfer functions

This document compares my results with those in the paper: ”A low-dimensional dynamic model of severe slugging for control design and analysis”.

The minimum peaks for the closed loop transfer functions is developed from the linearized 3-state model for two-phase-flow with an operating point for the choke-vale equal to 30% opening.

| Measurement | Scaling Dy | ¦G(0)¦ | ¦S¦=¦T¦ | ¦KS¦ |
| --- | --- | --- | --- | --- |
| P1 | 1 | 3.3(3.4) | 1.2(1) | 0.13(0.11) |
| DP | 0.5 | 6.2(6.3) | 1.6(1.9) | 0.33(0.25) |
| Rho_T | 50 | 0.21(0.26) | 1.3(1.5) | 0.18(0.13) |
| W | 1 | 0(0) | 1.0(1) | 0.08(0.06) |
| Q | 0.002 | 0.16 (0.28) | 1.2(1) | 0.13(0.09) |

The relationship between the scaled and unscaled model is  where  and  is given in the table above.

The reason for the deviations between E. Storkaas’ and my results is probably because of some differences in the parameters in the model.

The table below compares E. Storkaas’ results and mine with an operating point for the choke-valve equal to 17.5% opening.

| Measurement | Scaling Dy | ¦G(0)¦ | ¦S¦=¦T¦ | ¦KS¦ |
| --- | --- | --- | --- | --- |
| P1 | 1 | 16.4(19) | 1.1(1) | 0.01(0.01) |
| DP | 1 | 15.5(17.7) | 1.1(1.1) | 0.03(0.02) |
| Rho_T | 50 | 0.99(1.4) | 1.1(1.2) | 0.02(0.01) |
| W | 1 | 0(0) | 1.0(1) | 0.01(0.01) |
| Q | 0.002 | 0.76(1.5) | 1.1(1) | 0.02(0.01) |