Problem formulation:

y = Gu + Gdd

u = [Vfg Vlift]T		y = [CO2,sg Tr Fla]T		d = [d1 d2 d3]T

Controlled variables

- Cracking temperature (Tr)

- Flue gas oxygen concentration (CO2,sg)

## Associated variable

- Lift air compressor surge indicator (Fla)

Manipulated variables

- Lift air compressor speed (Vlift)

- Flue gas valve opening (Vfg)

Model disturbances

- Variations in ambient temp. – affect compressor throughput (d1)

- Fluctations in heavy oil feed composition to the FCCU (d2)

- Pressure upset in down stream units propagating back to the FCCU (d3)

# Running of  FFCU coloumn at different settings

| Simulation time (s) | tend = 2500 |
| --- | --- |
| No of step response coefficients | 60 |
| Process sampling time | delt2 = 100 |
| Output Weights | ywt = [3 3 0] |
| Input weights | uwt = [0 2] |
| Prediction Horizon times | P = 12 |
| No manipulated variable moves | M = 3 |
| input constraints | ui  [-1, 1] , i = 1,2 |
| output constraints | yi  [-1, 1] , i = 1,2 y3  -1 (hard constraint) |

Graph 1.: Starting values:

Uncontrained design:

- t=100s: controlled variabels outside their allowed limits. Onset of disturbance at t=0 unknown to the controller since there is no disturbance feedforward loop (no no control action between t=0 and t=100)  open loop response with no control action. After t=100 corrective action implemented.

- t=200 (2nd time step) riser temp. outside the allowed limits.

- Lift air surges during 200s  t  800s  unacceptable. Surging, outside the hard constraint, result in undesirable vibrations in compressor

Constrained design:

- t=100s: controlled variabels outside their allowed limits (same as for unconstrained design).

- t=200 (2nd time step) riser temp. outside the allowed limits. This is because the constrained QP solved at t=100s assumes that the disturbances are constant for t100s which is not the case for this process. Manipulated variable move made at t=100 ensures that predicted y2=1 at t=200s, the actual output at t=200s exceeds one.

- Lift air compressor does not surge during the disturbance transient.

Constrained/Unconstrained:

Unconstrained design violating hard constraint.

Parameters changed:

M from 3 to 1,2 and 4

Output weights from [3 3 0] to [1 1 0]

Input weights from [0 2] to [0 1]